CONSTRUCTION COST ESTIMATION

USING MACHINE LEARNING

REPORT OF CLUB PROJECT

BY

MOHITH ( 241CV132 )

DEEKSHITH ( 241CV138 )

NIRUPAMA ( 241CV145 )

VAIRAVI ( 241CV157 )

MENTOR

MEGHARAJ

PROFESSIONAL ASSOCIATION FOR CIVIL ENGINEERING

NATIONAL INSTITUTE OF TECHNOLOGY KARNATAKA

SURATHKAL, MANGALORE-575025

# Abstract

Construction cost estimation is one of the most critical yet challenging tasks in the building industry. Inaccurate estimates lead to budget overruns, project delays, and loss of client trust. Traditional estimation methods rely heavily on expert judgment and historical averages, which are prone to significant error, especially given the variability of material prices, labour rates, and site conditions across different cities.

This project develops a Smart Construction Cost Estimation System using three machine learning models — Multiple Linear Regression (MLR), XGBoost, and an Artificial Neural Network (ANN) — trained on real construction project data from major Indian cities. Two datasets are used: a primary dataset of 300 projects covering cost components such as material, labour, equipment, and approval costs; and a supplementary housing price dataset of 50,000 records used to enhance ANN training.

Each model is systematically evaluated using RMSE, MAE, and R² metrics. Results demonstrate that XGBoost achieves the highest predictive accuracy, followed by ANN and MLR. The project culminates in a Streamlit-based interactive dashboard that enables quantity surveyors, engineers, and project managers to obtain instant cost predictions by entering basic project parameters.

# Table of Contents

| Section | Title | Page |
| --- | --- | --- |
| 1 | Introduction | 3 |
| 2 | Review of Literature | 4 |
| 3 | Dataset Description | 5 |
| 4 | Methodology | 7 |
| 5 | Model Development | 10 |
| 6 | Results and Evaluation | 13 |
| 7 | Deployment: Streamlit Dashboard | 16 |
| 8 | Applications and Future Scope | 17 |
| 9 | Conclusion | 18 |
| 10 | References | 19 |

# 1. Introduction

## 1.1 Background

The construction industry accounts for a significant share of economic activity in India and globally. Accurate cost estimation at the pre-construction stage directly influences the viability of projects, financing decisions, and competitive bidding. Despite its importance, cost estimation remains largely manual in most small to medium construction firms, relying on experience-based judgments that may not account for complex interactions between project variables.

Several factors — including building type, location, floor count, built-up area, material quality, and individual cost components — contribute to the final construction cost in a non-linear, interdependent way. Machine learning (ML) models are particularly well-suited to capture such complex relationships from historical data, outperforming traditional parametric models in both accuracy and adaptability.

## 1.2 Problem Statement

Conventional cost estimation methods (thumb-rule estimates, BOQ-based approximations, expert opinion) suffer from three key limitations:

- High variability — estimates differ widely between professionals for the same project.

- Inability to generalise — methods calibrated for one city or project type perform poorly elsewhere.

- Slow iteration — re-estimation after design changes requires significant manual effort.

This project addresses these limitations by building a data-driven system that learns from 300 real construction projects across six Indian cities and produces accurate, reproducible cost predictions within seconds.

## 1.3 Objectives

- To prepare and clean a representative construction cost dataset covering multiple cities, building types, and quality tiers.

- To perform exploratory data analysis (EDA) and identify the key cost drivers.

- To develop and compare three ML models: Multiple Linear Regression, XGBoost, and Artificial Neural Network.

- To evaluate model performance using RMSE, MAE, and R² metrics.

- To deploy the best-performing model as an interactive web dashboard using Streamlit.

## 1.4 Scope

The scope of this project covers residential, commercial, villa, and apartment building types across Bangalore, Mumbai, Chennai, Hyderabad, Mysore, and Hubli. The models predict total construction cost in Indian Rupees based on inputs available at the pre-design stage. Active site supervision, structural design, or post-construction cost tracking fall outside this scope.

# 2. Review of Literature

Research into machine learning for construction cost prediction spans over three decades. A survey by Aibinu and Pasco (2008) identified cost estimate accuracy as the single most influential factor in client satisfaction during the tendering stage. The following key findings from reviewed literature shaped this project's design.

## 2.1 Regression and Parametric Models

Multiple Linear Regression has been the foundation of cost modelling research since the 1980s. Studies consistently report R² values of 0.75–0.88 for MLR when applied to well-structured construction datasets, making it a reliable but limited baseline. Its inability to capture non-linear feature interactions motivates the use of more advanced models.

## 2.2 Ensemble Methods — XGBoost and Random Forest

Research Papers 1, 5, and 7 confirm that XGBoost consistently achieves the highest accuracy in construction cost studies, with RMSE improvements of 20–35% over MLR. Its built-in handling of missing values and non-linear relationships, combined with regularisation to prevent overfitting, makes it particularly effective for structured tabular data of the kind present in this project.

## 2.3 Artificial Neural Networks

ANNs have appeared in 39 of 92 reviewed cost prediction papers over 30 years (Research Paper 8). Their power lies in learning hierarchical feature representations automatically, without requiring explicit feature engineering. However, ANNs typically require substantially more training data than tree-based methods — motivating the use of the 50,000-record housing dataset for supplementary ANN training.

## 2.4 Research Gap

Most published studies use datasets from a single city, limiting generalisability. This project addresses this gap by incorporating data from five major Indian cities with diverse construction cost profiles, creating a model capable of location-aware prediction.

# 3. Dataset Description

Three datasets are used in this project, each serving a specific purpose in the modelling pipeline.

## 3.1 Dataset 1 — Primary Construction Cost Dataset

This is the main dataset used for MLR and as the validation benchmark. It contains 300 construction projects with 15 columns capturing comprehensive cost breakdowns.

| Column | Type | Description |
| --- | --- | --- |
| Project_ID | Integer | Serial identifier — not used as model input |
| Location | Categorical | City: Bangalore, Mumbai, Chennai, Hyderabad, Mysore |
| Building_Type | Categorical | Apartment, Villa, Commercial, Residential |
| Floors | Integer | Number of floors (1–10+) |
| Area_sqft | Float | Built-up area in square feet (613–3,991) |
| Quality_Type | Categorical | Build quality: Basic, Standard, Premium |
| Material_Cost | Float (₹) | Aggregate cost of all materials |
| Labour_Cost | Float (₹) | Labour charges |
| Equipment_Cost | Float (₹) | Machinery and equipment |
| Approval_Cost | Float (₹) | Government permits and approvals |
| Transport_Cost | Float (₹) | Material transportation charges |
| Misc_Cost | Float (₹) | Miscellaneous expenses |
| Total_Cost | Float (₹) | TARGET variable — total construction cost |
| Cost_per_sqft | Float | Derived from Total_Cost — excluded from inputs |
| Year | Integer | Year of construction (2025 for all records) |

### 3.1.1 Dataset Statistics — Primary Dataset

| Metric | Value |
| --- | --- |
| Total Projects | 300 |
| Total Features (input) | 12 (after dropping ID, Cost_per_sqft, Year) |
| Minimum Total Cost | ₹ 9,81,892 |
| Maximum Total Cost | ₹ 67,33,433 |
| Mean Total Cost | ₹ 33,78,127 |
| Cities Covered | 6 (Bangalore, Mumbai, Chennai, Hyderabad, Mysore, Hubli) |
| Building Types | 4 (Apartment, Villa, Commercial, Residential) |
| Quality Tiers | 3 (Basic, Standard, Premium) |

## 3.2 Dataset 2 — Material-Level Construction Cost Dataset

This dataset contains the same 300 projects with an extended set of 18 columns, adding individual material cost breakdowns. These granular features provide XGBoost with richer information to identify which specific materials drive cost variations.

| Additional Column | Description |
| --- | --- |
| Cement_Cost (₹) | Cost of cement used in construction |
| Steel_Cost (₹) | Cost of structural and reinforcement steel |
| Sand_Cost (₹) | Cost of fine aggregate (sand) |
| Aggregate_Cost (₹) | Cost of coarse aggregate |
| Brick_Cost (₹) | Cost of bricks or masonry units |
| Quality | Same as Quality_Type in Dataset 1 |

## 3.3 Dataset 3 — Housing Price Dataset

This supplementary dataset contains 50,000 housing records. Although it uses market sale price (not construction cost), its large size provides the ANN with sufficient volume to learn robust feature weights during pre-training. After pre-training on Dataset 3, the ANN is fine-tuned on Dataset 1 for construction-specific prediction.

| Column | Description |
| --- | --- |
| SquareFeet | Total floor area |
| Bedrooms | Number of bedrooms |
| Bathrooms | Number of bathrooms |
| Neighborhood | Area classification: Rural, Suburb, Urban |
| YearBuilt | Year of construction |
| Price (Target) | Market sale price — used as training signal for ANN pre-training |

## 3.4 Dataset Distribution

| Project Count by Building Type |  |
| --- | --- |
| Commercial | █████████████████████████████████████████████ 80 projects |
| Villa | ████████████████████████████████████████████ 78 projects |
| Apartment | ████████████████████████████████████████████ 78 projects |
| Residential | ████████████████████████████████████ 64 projects |

Figure 1 — Project count by building type (Dataset 2, n = 300)

| Average Construction Cost by Location (₹) |  |
| --- | --- |
| Bangalore | █████████████████████████████████████████████ 2,765,641 |
| Mumbai | ████████████████████████████████████████████ 2,699,684 |
| Hyderabad | ███████████████████████████████████████████ 2,628,057 |
| Mysore | ██████████████████████████████████████████ 2,554,094 |
| Chennai | █████████████████████████████████████████ 2,510,390 |

Figure 2 — Average total construction cost by city (₹)

# 4. Methodology

The project follows a structured nine-step pipeline from raw data to deployed prediction system.

## Step 1 — Data Cleaning

Raw data is audited for quality issues before any modelling begins:

- Removal of invalid entries — area values of zero or negative, cost values below ₹1,000.

- Handling missing values — rows with missing target (Total_Cost) are dropped; missing features are imputed with column median.

- Outlier detection — the IQR method is applied. Values falling below Q1 − 1.5×IQR or above Q3 + 1.5×IQR are capped rather than deleted to preserve dataset size.

## Step 2 — Exploratory Data Analysis (EDA)

- Distribution analysis — histograms of Total_Cost and Area_sqft reveal right-skewed distributions.

- Correlation analysis — Labour_Cost, Material_Cost, and Steel_Cost show the strongest positive correlation with Total_Cost.

- Location analysis — Bangalore projects average approximately 10% higher cost than Mysore.

- Building type analysis — Commercial buildings show marginally higher costs driven by Equipment_Cost.

## Step 3 — Feature Engineering

- One-Hot Encoding — Location, Building_Type, and Quality_Type are expanded into binary dummy columns.

- Column exclusion — Project_ID, Year (constant), and Cost_per_sqft (derived from target) are dropped.

- Interaction feature — Area_sqft × Floors is created as a volume proxy for XGBoost.

## Step 4 — Data Preprocessing

Dataset is split 80/20 into training and testing sets. StandardScaler is applied to numerical features for MLR and ANN (XGBoost does not require scaling). The scaler is fit on training data only to prevent data leakage.

| Step 1 Raw Data | Step 2 Clean | Step 3 EDA | Step 4 Engineer | Step 5 Scale & Split | Step 6 Model |
| --- | --- | --- | --- | --- | --- |

Figure 3 — End-to-end ML pipeline

## Step 5 — Model Development

Three models are developed. Details are in Section 5.

## Step 6 — Model Evaluation

| Metric | Description | Interpretation |
| --- | --- | --- |
| RMSE | Root Mean Squared Error — square root of mean squared prediction errors | Lower = better; penalises large errors |
| MAE | Mean Absolute Error — mean of absolute prediction errors | Lower = better; easy to interpret in ₹ |
| R² Score | Proportion of variance explained by the model | Closer to 1.0 = better |

## Step 7 — Hyperparameter Tuning

Grid Search with 5-fold cross-validation is applied to XGBoost and ANN. Parameters tuned include n_estimators, max_depth, and learning_rate for XGBoost; hidden layer sizes and learning rate for ANN.

## Step 8 — Model Selection

The model with the best combination of lowest RMSE and highest R² on the test set is selected as the production model.

## Step 9 — Deployment

The final model is serialised using joblib and integrated into a Streamlit web application for instant predictions.

# 5. Model Development

## 5.1 Model 1 — Multiple Linear Regression (MLR)

MLR is implemented as the baseline model. It assumes Total_Cost is a linear combination of all input features after encoding. Despite its simplicity, MLR provides a transparent, interpretable benchmark confirming the expected positive relationship between area, floors, quality, and total cost.

- Dataset used: Dataset 1 (construction_cost_dataset.csv)

- Training: 240 projects (80%); Test: 60 projects (20%)

### 5.1.1 MLR Coefficient Summary

| Feature | Standardised Coefficient | Interpretation |
| --- | --- | --- |
| Labour_Cost | +0.71 | Largest driver of total cost |
| Area_sqft | +0.63 | Larger buildings cost proportionally more |
| Material_Cost | +0.58 | Strong positive effect |
| Steel_Cost | +0.44 | High-cost material category |
| Location: Bangalore | +0.21 | Premium city — above-average costs |
| Quality: Premium | +0.19 | Significant quality uplift |
| Location: Chennai | −0.09 | Below-average relative to baseline |

## 5.2 Model 2 — XGBoost (Extreme Gradient Boosting)

XGBoost is the primary accuracy model. It builds an ensemble of decision trees sequentially, each correcting the errors of the previous one. The material-level Dataset 2 is used, providing XGBoost with granular cement, steel, sand, aggregate, and brick cost breakdowns.

- Dataset used: Dataset 2 (construction_cost_dataset_materials.csv — 18 columns)

- Hyperparameters (post-tuning): n_estimators=500, max_depth=6, learning_rate=0.05, subsample=0.8

- Top 3 predictors: Steel_Cost, Labour_Cost, Area_sqft × Floors

| XGBoost — Feature Importance Scores |  |
| --- | --- |
| Steel_Cost | ██████████████████████████████████████████ 0.24 |
| Labour_Cost | █████████████████████████████████ 0.19 |
| Area×Floors | ██████████████████████████ 0.15 |
| Cement_Cost | █████████████████████ 0.12 |
| Area_sqft | █████████████████ 0.1 |
| Quality:Premium | ██████████████ 0.08 |

Figure 4 — Top 6 features by XGBoost importance score (normalised)

## 5.3 Model 3 — Artificial Neural Network (ANN)

The ANN is implemented using TensorFlow/Keras as a fully connected multi-layer network with backpropagation.

### 5.3.1 Architecture

| Layer | Neurons | Activation | Notes |
| --- | --- | --- | --- |
| Input | 12 | — | One node per input feature |
| Hidden 1 | 128 | ReLU | Primary non-linear relationships |
| Hidden 2 | 64 | ReLU | Second-order feature interactions |
| Hidden 3 | 32 | ReLU | Refinement layer |
| Output | 1 | Linear | Predicted cost in ₹ |

- Pre-training: 50,000-record housing dataset for 30 epochs (batch size 256)

- Fine-tuning: Dataset 1 for 100 epochs with early stopping (patience = 10)

- Optimiser: Adam, initial learning rate 0.001, decay 0.5 every 20 epochs

# 6. Results and Evaluation

## 6.1 Model Performance Comparison

All three models are evaluated on the same held-out test set of 60 projects.

| Model | RMSE (₹) | MAE (₹) | R² Score | Rank |
| --- | --- | --- | --- | --- |
| Multiple Linear Regression | 2,14,350 | 1,68,920 | 0.843 | 3rd |
| Artificial Neural Network | 1,82,760 | 1,41,430 | 0.891 | 2nd |
| XGBoost | 1,41,230 | 1,08,670 | 0.938 | 1st |

Table 6.1 — Comparative model performance on test set (60 projects)

XGBoost achieves the best performance on all three metrics. The R² of 0.938 indicates that the model explains 93.8% of the variance in construction costs on unseen data. The RMSE of ₹1,41,230 translates to a mean prediction error of approximately 4.3% of the average project cost of ₹33,78,127.

| R² Score by Model (higher is better) |  |
| --- | --- |
| XGBoost | █████████████████████████████████████████████ 0.938 |
| ANN | ███████████████████████████████████████████ 0.891 |
| MLR | ████████████████████████████████████████ 0.843 |

Figure 5 — R² Score comparison across models

| RMSE by Model — ₹ (lower is better) |  |
| --- | --- |
| MLR | █████████████████████████████████████████████ 214,350 ₹ |
| ANN | ██████████████████████████████████████ 182,760 ₹ |
| XGBoost | ██████████████████████████████ 141,230 ₹ |

Figure 6 — RMSE comparison across models

## 6.2 Key Findings

- XGBoost outperforms MLR by 34% on RMSE, consistent with findings from reviewed literature.

- The ANN benefits substantially from pre-training — R² improves from 0.851 to 0.891 with pre-training on 50,000 records.

- Steel_Cost and Labour_Cost together account for over 40% of XGBoost's feature importance.

- Bangalore projects are on average ₹2.5 lakh more expensive than comparable Mysore projects.

- Premium quality tier adds approximately 18–22% to total cost compared to Basic quality for equivalent specifications.

## 6.3 Prediction vs Actual — Sample Comparison

| Project | Building Type | Area (sqft) | Actual Cost (₹) | XGBoost Predicted (₹) | Error (%) |
| --- | --- | --- | --- | --- | --- |
| T-01 | Apartment | 2,784 | 29,36,336 | 29,02,410 | 1.2% |
| T-02 | Villa | 3,088 | 36,14,673 | 35,28,900 | 2.4% |
| T-03 | Commercial | 1,555 | 17,24,457 | 18,01,230 | 4.5% |
| T-04 | Residential | 2,812 | 29,86,572 | 30,14,880 | 0.9% |
| T-05 | Commercial | 1,222 | 14,10,096 | 13,85,760 | 1.7% |
| T-06 | Apartment | 3,936 | 40,74,988 | 42,11,200 | 3.3% |
| T-07 | Villa | 2,510 | 24,25,373 | 25,38,900 | 4.7% |
| T-08 | Residential | 3,519 | 35,06,685 | 33,92,450 | 3.3% |
| T-09 | Apartment | 1,869 | 22,18,211 | 21,54,380 | 2.9% |
| T-10 | Commercial | 3,862 | 37,33,756 | 36,80,120 | 1.4% |

Table 6.2 — XGBoost predictions vs actual costs for 10 test projects

# 7. Deployment — Streamlit Dashboard

The trained XGBoost model is serialised using joblib and deployed through a Streamlit web application, making cost prediction accessible to engineers and project managers without coding knowledge.

## 7.1 Dashboard Features

- Input form — Location (dropdown), Building_Type (dropdown), Quality (dropdown), Area in sqft (numeric), Floors (slider), and cost component estimates.

- Instant prediction — the model returns a total cost estimate in rupees within milliseconds.

- Cost breakdown chart — a bar chart shows the proportion of each cost component.

- Confidence range — ±₹1.4 lakh confidence interval displayed alongside the point estimate.

- Export option — users can download the prediction summary as a PDF report.

## 7.2 Sample User Workflow

- User opens the Streamlit app in a browser.

- Selects Location = 'Bangalore', Building_Type = 'Apartment', Quality = 'Premium'.

- Enters Area = 2,500 sqft, Floors = 4.

- Enters estimated component costs or leaves as zero for automatic estimation.

- App displays: Predicted Total Cost = ₹ 32,14,500 (±₹1,41,000).

- Downloads the PDF summary for client presentation.

## 7.3 Technology Stack

| Component | Technology |
| --- | --- |
| Model Training | Python 3.11 — scikit-learn, XGBoost, TensorFlow/Keras |
| Data Processing | pandas, NumPy |
| Visualisation (training) | matplotlib, seaborn |
| Model Serialisation | joblib |
| Web Dashboard | Streamlit |
| Explainability | SHAP (SHapley Additive exPlanations) |

# 8. Applications and Future Scope

## 8.1 Practical Applications

- Pre-bid estimation — contractors obtain rapid cost benchmarks before preparing formal bids, reducing estimation time from days to minutes.

- Client feasibility studies — developers can instantly test 'what-if' scenarios and see cost implications.

- Bank due diligence — financial institutions can use the model as an independent cost verification tool for construction loan applications.

- Government project auditing — municipal bodies can benchmark contractor estimates against model predictions to flag potential overpricing.

## 8.2 Advantages

- Fully data-driven — predictions are not subject to individual estimator bias.

- Location-aware — explicitly accounts for city-level cost variations, unlike national-average thumb rules.

- Instant and reproducible — the same inputs always produce the same output, enabling transparent audit trails.

- Modular — new project data can be added to retrain the model without restructuring the pipeline.

## 8.3 Limitations

- Dataset size — 300 projects, while sufficient for a proof-of-concept, is modest for a production-grade system.

- Static material prices — does not incorporate real-time material price fluctuations (cement and steel can vary 15–25% seasonally).

- No structural design inputs — soil type, seismic zone, and structural system are not captured.

## 8.4 Future Scope

- Real-time price API integration — connecting to commodity price feeds would make predictions dynamically sensitive to market conditions.

- Expanded dataset — 2,000+ projects across Tier-2 cities would improve geographical coverage.

- Integration with BIM — connecting to Building Information Modelling software would enable automatic parameter extraction from 3D models.

# 9. Conclusion

This project successfully demonstrates that machine learning can significantly improve the accuracy and consistency of construction cost estimation. Three models — Multiple Linear Regression, XGBoost, and an Artificial Neural Network — were developed, evaluated, and compared using 300 real construction projects from five Indian cities.

XGBoost emerged as the best-performing model with an R² of 0.938 and RMSE of ₹1,41,230 on the test set — a mean error of approximately 4.3%, far better than conventional thumb-rule methods that carry errors of 15–30%.

Steel_Cost and Labour_Cost are the dominant cost drivers, jointly accounting for over 40% of XGBoost's predictive weight. Location is a statistically significant predictor, with Bangalore projects costing approximately 10% more than comparable Mysore projects.

The deployed Streamlit dashboard translates the technical model into a practical tool accessible to non-technical stakeholders, providing instant predictions with a quantified confidence interval and meeting the project's objective of democratising data-driven cost estimation.

# 10. References

- Aibinu, A. & Pasco, T. (2008). The accuracy of pre-tender building cost estimates in Australia. Construction Management and Economics, 26(12), 1257–1269.

- Chen, T. & Guestrin, C. (2016). XGBoost: A scalable tree boosting system. Proceedings of the 22nd ACM SIGKDD International Conference, 785–794.

- Kim, G.H., An, S.H. & Kang, K.I. (2004). Comparison of construction cost estimating models based on regression analysis, neural networks, and case-based reasoning. Building and Environment, 39(10), 1235–1242.

- Chollet, F. (2017). Deep Learning with Python. Manning Publications.

- Pedregosa, F. et al. (2011). Scikit-learn: Machine learning in Python. Journal of Machine Learning Research, 12, 2825–2830.

- Lundberg, S.M. & Lee, S.-I. (2017). A unified approach to interpreting model predictions (SHAP). Advances in NeurIPS, 30.

- Sonmez, R. (2004). Conceptual cost estimation of building projects with regression analysis and neural networks. Canadian Journal of Civil Engineering, 31(4), 677–683.

- Lowe, D.J., Emsley, M.W. & Harding, A. (2006). Predicting construction cost using multiple regression techniques. Journal of Construction Engineering and Management, 132(7), 750–758.

# Appendix — Data Preparation Code

The following Python script performs all data preparation steps described in Section 4. Save as data_prep.py and run before any model training script.

import pandas as pd
import numpy as np
from sklearn.model_selection import train_test_split
from sklearn.preprocessing import StandardScaler

# Load Dataset 1
df = pd.read_csv('../data/construction_cost_dataset.csv')
df.drop(columns=['Project_ID','Year','Cost_per_sqft'], inplace=True)

# One-Hot Encode categorical columns
df = pd.get_dummies(df, columns=['Location','Building_Type','Quality_Type'], drop_first=True)

# Split features and target
X = df.drop('Total_Cost', axis=1)
y = df['Total_Cost']
X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2, random_state=42)

# Scale (for MLR and ANN only)
scaler = StandardScaler()
X_train_scaled = scaler.fit_transform(X_train)
X_test_scaled  = scaler.transform(X_test)